Please read the following notes before proceeding

- It is recommended to save the project on the C drive during development. Once you have completed your work, you must save the project inside the exam folder on the Z drive (the same folder where you found this document). Projects saved elsewhere will not be collected and will be permanently lost.

- The assembly subroutine developed in the first exercise must comply with the ARM Architecture Procedure Call Standard (AAPCS) standard (in terms of parameter passing, returned value, callee-saved registers).

- If you prefer, you can begin with the second exercise. Since it requires calling the assembly subroutine from the first exercise, you can create a stub in the assembly file to proceed. For example:

subroutineName	PROC

; to do: add code

MOV r0, #10		; example of a return value

BX LR

ENDP

- To earn points on all exercises, it is recommended that you write at least some code for each.

- The final project must run on the actual board (e.g., button debouncing is required).

Special characters with Italian keyboards

- Square brackets [ ]
 	Left bracket:	ATL GR + [

Right bracket:	ALT GR + ]

- Curly brackets { }

Left bracket:	LSHIFT + ALT GR + [

Right bracket:	LSHIFT + ALT GR + ]

- Hash # :	ALT GR + à

Bitwise operations in C

Left shift by N bits 	= 	A<<N     
Right shift by N bits 	= 	A>>N

A XOR B 	= 	A^B

A AND B 	= 	A&B  	(&& is the logical AND)

A OR B 	= 	A|B 	(|| is the logical OR)

Question 1 (6 points)

The aliquot sum s(n) of a natural numbers n is the sum of all proper divisors of n, that is, all divisors of n other than n itself. Examples:

- the aliquot sum of 28 is: s(28) = 1 + 2 + 4 + 7 + 14 = 28.

- the aliquot sum of 102 is: s(102) = 1 + 2 + 3 + 6 + 17 + 34 + 51 = 114

- the aliquot sum of 158 is: s(158) = 1 + 2 + 79 = 82

A natural number lower than its aliquot sum is called a deficient number.

A natural number equal to its aliquot sum is called a perfect number.

A natural number greater than its aliquot sum is called an abundant number.

In the examples above, 28 is perfect, 102 is deficient, and 158 is abundant.

Write the aliquotSum subroutine in ARM assembly language, which receives a natural number n in input, and returns:

- 0 if n is deficient

- 1 if n is perfect

- 2 if n is abundant

Question 2 (12 points)

Add the following functionalities to your project.

1) Start by initializing a variable to 0. The user will set the value of this variable using the buttons.

- Each time Int0 is pressed, add 1 to the variable.

- Each time Key2 is pressed, add 10 to the variable.

- Each time Key1 is pressed, add 100 to the variable.
The buttons can be pressed repeatedly, in any order, so the user can build up any number they want.

2) Once the desired number has been entered, the user confirms their choice by pressing the joystick (select function).

3) At this point, the program calls the aliquotSum subroutine, passing the number entered by the user. Based on the result, one of the LEDs will be turned on to indicate the classification of the number:

- If the number is deficient, LED11 is turned on.

- If the number is perfect, LED10 is turned on.

- If the number is abundant, LED9 is turned on.

4) Finally, when the classification has been displayed, the program generates a short sound to signal that the computation is complete. The tone and duration of the sound are up to you, but it should last no longer than 2 seconds. After the sound is played, reset the variable back to 0 so the user can start again.